Computer Architecture and Organization

Digital Assignment 3

Name: Harshit Kakati

Reg No. 24BCE1450

Prof Dr. Abdul Quadir

Slot: E2+TE2

Aim: To execute Project 3 and Project 4

Project 3:

- Single Bit ram:

Code:

CHIP Bit {

IN in, load;

OUT out;

PARTS:

Mux(a=dffOut, b=in, sel=load, out=muxOut);

DFF(in=muxOut, out=dffOut);

Or(a=false, b=dffOut, out=out);

}

- Register 16 bit:

CHIP Register {

IN in[16], load;

OUT out[16];

PARTS:

Bit(in=in[0], load=load, out=out[0]);

Bit(in=in[1], load=load, out=out[1]);

Bit(in=in[2], load=load, out=out[2]);

Bit(in=in[3], load=load, out=out[3]);

Bit(in=in[4], load=load, out=out[4]);

Bit(in=in[5], load=load, out=out[5]);

Bit(in=in[6], load=load, out=out[6]);

Bit(in=in[7], load=load, out=out[7]);

Bit(in=in[8], load=load, out=out[8]);

Bit(in=in[9], load=load, out=out[9]);

Bit(in=in[10], load=load, out=out[10]);

Bit(in=in[11], load=load, out=out[11]);

Bit(in=in[12], load=load, out=out[12]);

Bit(in=in[13], load=load, out=out[13]);

Bit(in=in[14], load=load, out=out[14]);

Bit(in=in[15], load=load, out=out[15]);

}

- Ram 8 Bit:

CHIP RAM8 {

IN in[16], load, address[3];

OUT out[16];

PARTS:

DMux8Way(in=load, sel=address,

a=sel0,

b=sel1,

c=sel2,

d=sel3,

e=sel4,

f=sel5,

g=sel6,

h=sel7

);

Register(in=in, load=sel0, out=r0);

Register(in=in, load=sel1, out=r1);

Register(in=in, load=sel2, out=r2);

Register(in=in, load=sel3, out=r3);

Register(in=in, load=sel4, out=r4);

Register(in=in, load=sel5, out=r5);

Register(in=in, load=sel6, out=r6);

Register(in=in, load=sel7, out=r7);

Mux8Way16(

a=r0,

b=r1,

c=r2,

d=r3,

e=r4,

f=r5,

g=r6,

h=r7,

sel=address,

out=out);

}

- Ram 64 Bit:

CHIP RAM64 {

IN in[16], load, address[6];

OUT out[16];

PARTS:

DMux8Way(in=load, sel=address[3..5],

a=sel0,

b=sel1,

c=sel2,

d=sel3,

e=sel4,

f=sel5,

g=sel6,

h=sel7

);

RAM8(in=in, load=sel0, address=address[0..2], out=r0);

RAM8(in=in, load=sel1, address=address[0..2], out=r1);

RAM8(in=in, load=sel2, address=address[0..2], out=r2);

RAM8(in=in, load=sel3, address=address[0..2], out=r3);

RAM8(in=in, load=sel4, address=address[0..2], out=r4);

RAM8(in=in, load=sel5, address=address[0..2], out=r5);

RAM8(in=in, load=sel6, address=address[0..2], out=r6);

RAM8(in=in, load=sel7, address=address[0..2], out=r7);

Mux8Way16(

a=r0,

b=r1,

c=r2,

d=r3,

e=r4,

f=r5,

g=r6,

h=r7,

sel=address[3..5],

out=out);

}

- PC(Program Counter):

CHIP PC {

IN in[16],inc, load, reset;

OUT out[16];

PARTS:

Inc16(in=regOutput, out=regOutputInc);

Mux16(a=regOutput, b=regOutputInc, sel=inc, out=out1);

Mux16(a=out1, b=in, sel=load, out=out2);

Mux16(a=out2, b=false, sel=reset, out=regInput);

Or(a=load, b=inc, out=loadOrInc);

Or(a=loadOrInc, b=reset, out=loadOrIncOrReset);

Register(in=regInput, load=loadOrIncOrReset, out=regOutput);

Or16(a=false, b=regOutput, out=out);

}

- RAM 4K:

CHIP RAM4K {

IN in[16], load, address[12];

OUT out[16];

PARTS:

DMux8Way(in=load, sel=address[9..11],

a=sel0,

b=sel1,

c=sel2,

d=sel3,

e=sel4,

f=sel5,

g=sel6,

h=sel7

);

RAM512(in=in, load=sel0, address=address[0..8], out=r0);

RAM512(in=in, load=sel1, address=address[0..8], out=r1);

RAM512(in=in, load=sel2, address=address[0..8], out=r2);

RAM512(in=in, load=sel3, address=address[0..8], out=r3);

RAM512(in=in, load=sel4, address=address[0..8], out=r4);

RAM512(in=in, load=sel5, address=address[0..8], out=r5);

RAM512(in=in, load=sel6, address=address[0..8], out=r6);

RAM512(in=in, load=sel7, address=address[0..8], out=r7);

Mux8Way16(

a=r0,

b=r1,

c=r2,

d=r3,

e=r4,

f=r5,

g=r6,

h=r7,

sel=address[9..11],

out=out);

}

- RAM 512 Bit:

CHIP RAM512 {

IN in[16], load, address[9];

OUT out[16];

PARTS:

DMux8Way(in=load, sel=address[6..8],

a=sel0,

b=sel1,

c=sel2,

d=sel3,

e=sel4,

f=sel5,

g=sel6,

h=sel7

);

RAM64(in=in, load=sel0, address=address[0..5], out=r0);

RAM64(in=in, load=sel1, address=address[0..5], out=r1);

RAM64(in=in, load=sel2, address=address[0..5], out=r2);

RAM64(in=in, load=sel3, address=address[0..5], out=r3);

RAM64(in=in, load=sel4, address=address[0..5], out=r4);

RAM64(in=in, load=sel5, address=address[0..5], out=r5);

RAM64(in=in, load=sel6, address=address[0..5], out=r6);

RAM64(in=in, load=sel7, address=address[0..5], out=r7);

Mux8Way16(

a=r0,

b=r1,

c=r2,

d=r3,

e=r4,

f=r5,

g=r6,

h=r7,

sel=address[6..8],

out=out);

}

- RAM 16K:

CHIP RAM16K {

IN in[16], load, address[14];

OUT out[16];

PARTS:

DMux4Way(in=load, sel=address[12..13],

a=sel0,

b=sel1,

c=sel2,

d=sel3

);

RAM4K(in=in, load=sel0, address=address[0..11], out=r0);

RAM4K(in=in, load=sel1, address=address[0..11], out=r1);

RAM4K(in=in, load=sel2, address=address[0..11], out=r2);

RAM4K(in=in, load=sel3, address=address[0..11], out=r3);

Mux4Way16(

a=r0,

b=r1,

c=r2,

d=r3,

sel=address[12..13],

out=out);

}

Project 4:

- Fill the screen black:

Code:

@color    // declare color variable

M=0      // by default is white

(LOOP)

@SCREEN

D=A

@pixels

M=D         // pixel address, goes from 16384 to 16384 + 8192 == 24576

@KBD    // keyboard address

D=M

@BLACK

D;JGT     // if(keyboard > 0) goto BLACK

@color

M=0       // set to white

@COLOR_SCREEN

0;JMP     // jump to subroutine that colors the screen

(BLACK)

@color

M=-1    // set to black (2's complement 111111111...)

(COLOR_SCREEN)

@color

D=M

@pixels

A=M         // VERY IMPORTANT! indirect address

M=D         // color M[pixels] with @color

@pixels

M=M+1

D=M

@24576

D=D-A

@COLOR_SCREEN

D;JLT

@LOOP

0;JMP // infinite loop

Execution:

- Multiplication:

Code:

@R2

M=0

(LOOP)

@R1

D=M

@END

D;JEQ   // if(R1==0) goto END

@R0

D=M       // D = R0

@R2

M=M+D     // R2 = R2+ R0

@R1

M=M-1     // R1--

@LOOP

0;JMP     // goto LOOP

(END)

@END

0;JMP // infinite loop